# Gearbox ratios	(Honda)	(mostly from Glass’s Guide TSD – some of it of dubious accuracy)

# PG1 gearboxes		5	4	3	2	1	final	spread 	type		mph/1000	etc.

MG 		1985’86	0.85	1.04	1.33	1.89	3.25	3.875	3.823	G6AR ?

216 EFi	1990	0.85	1.04	1.33	1.89	3.25	3.87	ditto

820 e + i	1987’88	0.848	1.033	1.307	1.894	3.250	3.937	3.833

Elise 111S	2002	0.85	1.03	1.31	1.89	3.25	3.77	3.823		Autocar

MG Montego	1987’89	0.77	0.96	1.24	1.75	2.92	3.937	3.792	K6AR

MG Maestro 	1990-91	0.77	0.96	1.24	1.75	2.92	3.975	3.792	K6AR/AO

220 M16	138	1991	0.765	0.935	1.222	1.750	2.923	4.200	3.821	K4	22 (w.man)

220 T16	136	1992-95	0.765	0.935	1.222	1.750	2.923	4.200	3.821	K4BS	22

420 T16	136	1992-95	0.765	0.935	1.222	1.750	2.923	3.938	3.821	K6BS	22

420 i		136 	1997-99	0.765	0.935	1.222	1.750	2.923	3.937	3.821	K6BN

MG Maestro	1987’89	0.76	0.94	1.22	1.75	2.92	3.87	3.842	K6AR

MG Montego	1990-92	0.76	0.94	1.22	1.75	2.92	3.937	3.842	K6AO

MG turbo	150	1990-92	0.76	0.94	1.22	1.75	2.92	3.647	3.842	K7 AO?	25

220 t		200	1993-95	0.765	0.935	1.222	1.750	2.923	3.647	3.821	K7 BX?				**

MG Maestro	1992	0.75	0.94	1.22	1.75	2.92	3.937	3.893	K6AO	23

MG TF 135		2002	0.76	1.03	1.23	1.84	3.17	4.2				Autocar

218 iS		120	1999	0.765	1.033	1.308	1.842	3.167	3.938	4.140	C6BS

200 Vi		143 	1997-99	0.765	1.033	1.308	1.842	3.167	4.200	4.140	C4BS

VVC Coupe		1997-99	+ 25GTi  2000  ditto all

620 ti		200 	1997-99	0.765	1.033	1.308	1.842	3.167	3.938 	4.140	C6FTUT			 	**

MGF		120	1999	0.765	1.033	1.308	1.842	3.167	3.937 	4.140	C6BP				****

MGF vvc	145	1999	0.765	1.033	1.308	1.842	3.167	4.200	4.140	C4BP

825 KV6		1997	0.765	1.033	1.308	1.842	3.167	3.937	4.140	C6DTUTH	TSD

820 e + i M16	1989-92	0.764	0.935	1.307	1.894	3.250	3.937	4.254	V4DT	24.1	(w.man)

820 i	T16	136	1992-97	0.764	0.935	1.222	1.894	3.250	4.20	4.254	V4DT

820 t		180/200	1994-97	0.764	0.935	1.222	1.894	3.250	3.90 ??	4.254	R4DT     				**

218 D		66/67	1992-95	0.764	0.935	1.222	1.894	3.250	3.937	4.254	V6BS

218 Dt		88	1992-95	0.764	0.935	1.222	1.894	3.250	4.062	4.254	M5BS	TSD	  ***

218 Dt			1991	0.648	0.848	1.222	1.842	3.166	4.062	4.885	M5BS 	w. man

Montego td	1990-93	0.65	0.85	1.22	1.84	3.166	4.062 ??	4.870	M7AO	own G-reg.

Maestro D van	1989	0.65	0.85	1.22	1.84	3.166	4.062 	4.870	M5AO ?	TSD

216 S		1987	0.66	0.87	1.15	1.75	2.92	4.1	4.424	L5 ?

Montego carb *	1987-93	0.65	0.85	1.14	1.75	2.92	3.937	4.492	L6AR	E-reg. sticker

Montego td	1989	0.65	0.85	1.14	1.75	2.92	3.937	4.492	L6 ?		TSD

Maestro D van	1991’92	0.714	0.906	1.222	1.894	3.25	4.200	4.552	T4AO	TSD

Montego 1.6	1989’92	0.714	0.906	1.222	1.894	3.25	4.062	4.552	T5AR	Haynes

220 D		86 	1997-99	0.648	0.848	1.222	1.894	3.250	3.938	5.015	S6BS

220 Di	 	105 	1997-00	0.648	0.848	1.222	1.894	3.250	3.938	5.015	S6BSU

420 D		86 	1997-99	0.648	0.848	1.222	1.894	3.250	3.938	5.015	S6BN

420 Di		105 	1997-99	0.648	0.848	1.222	1.894	3.250	3.938	5.015	S6BNU

620 Di		105 	1997-99	0.648	0.848	1.222	1.894	3.250	4.200	5.015	S4FTU

Montego td		K-reg. own	0.648	0.848	1.222	1.894	3.250	3.938	5.015	S6AO 	sticker

*	including some EFi cars  –  i.e. those without MG or  GTi badges.

3.647 	= 62/17							coded 	7 above, apparently

3.77	= 64/17 (3.7647)

3.875 	= 62/16		 3.87 	believed to be 3.875			6  ??

3.937 	= 63/16 (3.9375)	 3.975 / 3.397 / 3.90 	believed to be 3.9375	6		3.90 could be 78/20

4.062 	= 65/16 (4.0625)	 4.1 	believed to be 4.0625 			5

4.200 	= 63/15								4

If this theory is right, then clearly some of the above final drive data may be wrong, e.g. Montego td with M7AO, should have 3.6 f.d. Turbo engines also have gearboxes with uprated bearings.

** 16-valve turbos believed to have a Torsen diff., part no. TBB 100550.

**** 45 1.8 (2000) has identical spec.

## K type ratios

MG Maestro and Montego between 87 and 92 seem to be all mixed up, in terms of ratios. I suspect that all the K ratio sets are actually the same, and that the publications are wrong regarding some of them. This probably goes right back to the workshop manuals, which don’t always seem to be right. And Glass’s probably take their data from there. I believe the actual ratios are as detailed for the T16 models, with three decimal places.

Group 1 listed above is supposed to be G6AR, according to the website piece about gearboxes, but all my Maestros, even the B-reg. ones, had K6AR boxes. I guess the spec. changed at an earlier time than the TSD books suggest. Incidentally, the Haynes manual says that the G6 ratios remained throughout the life of the MG Montego. Clearly wrong! My guess is that the very early 2.0 cars had the G6 box, and that it was changed to the K6 around the time of the B registrations. Alternatively, my two B-reg. boxes may not be original, but may be later boxes put into the earlier cars at some time. Improbable! Or maybe all of the above codes are wrong?

## V and R ratios

M16 820 has V4DT sticker (2 versions of it, F and G-reg.). T16 820 has R4DT sticker. There is no obvious difference between them, either visually or in terms of the above data. However, the TSD book reckons that the R box is fitted to the 820 turbo, in which case it may have an LSD. It would be a nice stroke of luck if it did, but knowing my luck, it won’t! The 4 in R4DT suggests it has a 4.2 f.d. rather than the one shown in TSD (3.90, which doesn’t make sense).

The manual says the M16 820 should have a V4DT box, and sure enough, that is confirmed by my two boxes. I also have two earlier 820s, but they don’t have the stickers on the bell-housings, so I can’t identify them in this way.

V4DT may well be a better box for a future M16 or T16 conversion – slightly lower first and second than the K6 box in the Montego. Slight discrepancy in third. Not sure, but suspect the above 1.307 may be wrong. Mind you, it did come from the 820 factory manual. But they have been known to be wrong. This is not important. But V6 or R6 even better with 3.9.

## M ratios

*** 218 td. Glass’s Guide TSD says one thing – factory workshop manual says the other. I am inclined to believe the manual in this case. The M would agree with the other Ms, in terms of intermediate ratios. Similarly, the number is consistent with the final drive theory if you believe the workshop manual. So the first line of the two is suspect. I think all the turbo-diesels should have the M ratios, not the ones suggested in the *** line.

## Montego diesels

These seem to fall into three different categories. I suspect there is something wrong here. The 1989 TSD book says one thing, and later books say another thing. Actual stickers on boxes say something else again. Confusing!!

I have two Montego turbo-diesels, and their gearboxes are different, in respect of the codes at least. Both are shown above. Does this mean that they have different f.d. ratios? Or is one a non-standard box? Neither number is consistent with my theory about the f.d. ratio. The S6AO is in the K-reg. Td estate. M7AO is in the G-reg. Td saloon. I have guessed at the former’s spec. in the second line of the Montego td. I suspect that the f.d. ratios are wrong in the TSD books, and that these ratios should be as per the theory outlined above, i.e. 4 = 3.647, etc. If this is the case, then S6AO will be 3.9375 and M7AO will be 3. 647. This will make the G-reg. Saloon very high geared.

## Maestro van

T4AO will be 4.200. The Maestro van does have bigger tyres, so this would be consistent.

# TSD = Glass’s Guide Technical Service Data

# Actual stickers found on vehicles

Maestro van 		J-reg.		T4AO

Montego td Estate 	K		S6AO	x 2

Montego td saloon	G		M7AO

820i 	M16		F		V4DT

G		V4DT

820i	T16		K		R4DT

MG Maestro		B		K6AR	x 2

F		K6AR

G		K6AO

H		K6AO

MG Montego		G		K6AO

Montego carb.		E		L6AR

Montego V-P (EFi)	E		L6AR

420 Estate		N		K6BS

There is some confusion over V and R ratio sets. Here is some more detail. The first line is data from the workshop manual for the 820 M16 (complete with road speed data).

820 e + i M16	1989-92	0.764	0.935	1.307	1.894	3.250	3.937		(w. man.)		V4DT

mph/1000	24.1	19.7	14.1	9.7	5.7

This goes against the general trend regarding final drive ratios – i.e. 4 means 4.2, 6 means 3.9375, etc. So I am inclined not to believe it, although the road speed data would suggest the 3.9 ratio may be right. Mind you, even the 4.2 f.d. may give this road speed with the bigger wheels on the 820.   3.937 / 4.2  =  0.9375.  Reciprocal  =  1.06*. This means a 6.66666% increase in overall gearing between the 4.2 and 3.9 f.d. ratios.

820 i	T16	136	1992	0.764	0.935	1.307	1.894	3.250	3.937			TSD		V4DT

1993-97	0.764	0.935	1.222	1.894	3.250	4.20			TSD		V4DT

820 t		180/200	1992	0.764	0.935	1.222	1.894	3.250	4.20			TSD		not specified

1993-97	0.765	1.033	1.307	1.894	3.250	3.90			TSD		R4DT

820 i	T16	136	1993	K 586 XUD							sticker		R4DT

This is all very inconsistent. I am inclined to believe that the turbos will be 3.9, and the non-turbos 4.2. But the figure 4 in these codes would suggest they are all 4.2. But the factory workshop manual says 3.9, and the Glass’s Guide TSD books give all sorts of confusing details.

Tyre sizes:	  195/65 x 15, 820 = 62 cm;  185/65 x 14 Montego = 57 cm;  185/60 Maestro = 56 cm  (approx. diameter)

Lotus Elise is also believed to have a Honda gearbox. No data available in TSD. Demon Tweeks says it has the same LSD application as the 220. Logical, as it has an MGF engine! Some Hondas may have the same type of box, but I haven’t been able to identify any yet. I suspect not, however, as only the Legend has the gearbox at the same side of the car, to the best of my knowledge.

## PG2 gearbox  –  don’t know what the distinction is. Slightly stronger in a few places, I think. Probably shared with Honda Legend.

827		1990	0.702	0.909	1.222	1.789	2.923	4.200	4.164			G3U4

Gear ratios

1st

2.923076923		38/13

3.166666667		38/12

3.250			39/12

2nd

1.750			35/20

1.842105263		35/19

1.894736842		36/19

3rd

1.142857143		32/28

1.15			ditto ?	or 	23/20

1.222222222		33/27

1.24			31/25

1.307692308		34/26

1.333333333		32/24

4th

0.848484848		28/33

0.85			34/40	or	same as above

0.90625			29/32

0.909090909		30/33

0.935483871		29/31

0.94			ditto ?

0.96			24/25

1.033333333		31/30

1.04			26/25

5th

0.648648649		24/37

0.65			26/40 	or	same as above

0.666666666		28/42	or 	14/21?

0.714285714		25/35

0.75			24/32

0.764705882		26/34

0.77			ditto ?

0.848484848		28/33

0.85			34/40	or	same as above

K16 1.8 engines use the Honda-type gears, but the casings are different from those used with the various 2-litre engines. All O, M and T-Series gearboxes are interchangeable with each other, including the diesels, I believe. But the K-Series ones aren’t interchangeable with these as complete assemblies, because of the different bell-housing, as far as I know.

# Observations

MG Maestro is spot-on for gearing.

M16 Montego is a fraction high in 1st for taking off. 185/60 tyres might be enough to correct it, but I feel a slightly wider ratio set would be better, e.g. C, V or R ratios.